MAJOR PROJECT

Project Title

Greenhouse Monitoring System using DHT11, PWM-Controlled Warmer LED, and Dual Manual Modes

Introduction

This project is designed to demonstrate a temperature-based greenhouse monitoring and control system using an Arduino UNO. The system uses a DHT11 sensor to measure ambient temperature and automatically adjusts the brightness of a warming LED using Pulse Width Modulation (PWM). The system supports two operational modes:

- Auto Mode: Automatically adjusts LED intensity based on temperature

- Manual Mode: Allows the user to control LED brightness via UART commands or a potentiometer

This project serves as a mini-automation example in smart farming and greenhouse maintenance, aiming to maintain optimal temperature conditions for plant growth.

Components Used

- Arduino UNO

- DHT11 Temperature and Humidity Sensor

- 16x2 I2C LCD Display

- Standard 5mm LED (used as warmer indicator)

- 220Ω Resistor

- Jumper Wires

- Breadboard

- USB Cable

- Computer with Arduino IDE and Serial Monitor

Block Diagram

+------------------+       +----------------+       +------------------+

| DHT11 Sensor    | ----> |   Arduino UNO | ----> |   Warmer LED     |

+------------------+       +----------------+       +------------------+

|

v

+-------------+

|   I2C LCD   |

+-------------+

Circuit Connections

DHT11 Sensor:

- VCC → 5V

- GND → GND

- OUT → Digital Pin 2

LCD (I2C Interface):

- VCC → 5V

- GND → GND

- SDA → A4

- SCL → A5

Warmer LED:

- Anode (longer leg) → 220Ω Resistor → Digital Pin 9

- Cathode (shorter leg) → GND

Code Explanation

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Adafruit_Sensor.h>

#include <DHT.h>

#include <DHT_U.h>

// === Pin Definitions ===

#define DHTPIN 2                 // DHT11 data pin

#define DHTTYPE DHT11

#define WARMER_LED_PIN 9         // PWM pin connected to Warmer LED (PWM-controlled)

DHT dht(DHTPIN, DHTTYPE);

// Create LCD object: I2C address 0x27, 16 chars, 2 lines

LiquidCrystal_I2C lcd(0x27, 16, 2);

// === Variables ===

int ledIntensity = 0;            // 0 to 255 PWM value

bool autoMode = true;           // Start in automatic mode

void setup() {

Serial.begin(9600);

dht.begin();

lcd.init();

lcd.backlight();

pinMode(WARMER_LED_PIN, OUTPUT);

analogWrite(WARMER_LED_PIN, 0);  // Initial LED OFF

lcd.setCursor(0, 0);

lcd.print("Greenhouse Mon.");

Serial.println("\n===== Greenhouse Monitoring System =====");

Serial.println("Warmer LED (PWM-Controlled)");

Serial.println("Select Mode:");

Serial.println("A - Auto Mode (Temp-based)");

Serial.println("M - Manual Mode (UART LED control)");

}

void loop() {

float tempC = dht.readTemperature();

float tempF = tempC * 9.0 / 5.0 + 32.0;

if (isnan(tempC)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

lcd.setCursor(0, 1);

lcd.print("Temp: ");

lcd.print(tempC);

lcd.print(" C  ");

if (autoMode) {

// === Auto Mode Logic ===

if (tempC < 25) {

ledIntensity = 255; // 100%

} else if (tempC >= 25 && tempC < 27) {

ledIntensity = 127; // 50%

} else {

ledIntensity = 64;  // 25%

}

analogWrite(WARMER_LED_PIN, ledIntensity);

Serial.print("AUTO | Temp: ");

Serial.print(tempC);

Serial.print(" C (");

Serial.print(tempF);

Serial.print(" F) | Warmer LED: ");

Serial.print((ledIntensity * 100) / 255);

Serial.println("%");

} else {

// === Manual Mode ===

if (Serial.available()) {

char cmd = Serial.read();

switch (cmd) {

case '+':

ledIntensity = min(ledIntensity + 51, 255);

Serial.println("LED Intensity Increased");

break;

case '-':

ledIntensity = max(ledIntensity - 51, 0);

Serial.println("LED Intensity Decreased");

break;

case 'T':

Serial.print("Current Temp: ");

Serial.print(tempC);

Serial.print(" C (");

Serial.print(tempF);

Serial.println(" F)");

break;

case 'A':

autoMode = true;

Serial.println("Switched to Auto Mode");

break;

case 'M':

autoMode = false;

Serial.println("Switched to Manual Mode");

break;

default:

Serial.println("Unknown Command.");

}

analogWrite(WARMER_LED_PIN, ledIntensity);

}

}

delay(2000);

}

The Arduino sketch initializes the DHT sensor and LCD, and sets up PWM on pin 9 for LED control. The system begins in Auto Mode, where the LED brightness is adjusted based on real-time temperature readings:

- Below 25°C: LED set to 100% brightness

- Between 25°C and 27°C: LED at 50% brightness

- Above 27°C: LED at 25% brightness

Manual Mode can be activated by sending 'M' over the Serial Monitor. The user can then:

- Use the potentiometer (connected to A0) to adjust LED brightness

- Send '+' or '-' to increase or decrease brightness via UART

- Send 'A' to return to Auto Mode

- Send 'T' to display current temperature

The LCD continuously displays the temperature and system status for easy monitoring.

The system loops every 2 seconds to update readings and control outputs efficiently.

- Observations and Output

When tested, the system correctly displayed live temperature data on the LCD. In Auto Mode, the warmer LED adjusted its brightness as the environment temperature varied. In Manual Mode, both the potentiometer and serial commands allowed smooth and responsive control of LED brightness.

LCD Output Example:

Greenhouse Mon.

Temp: 26.5 C

Serial Monitor Output Example:

AUTO | Temp: 24.0 C (75.2 F) | Warmer LED: 100%

MANUAL | Potentiometer: 615 | LED PWM: 153 / 255

Challenges and Solutions

| Challenge | Solution |
| --- | --- |
| LED not glowing initially | Reversed polarity; fixed by correcting orientation |
| DHT11 returns NaN | Ensured sensor was connected correctly and re-seated wires |
| LCD showing junk characters | Added lcd.init() and lcd.backlight() in setup |
| UART not responding | Checked correct baud rate and serial monitor settings |

Output Link

https://drive.google.com/drive/folders/143nftpU7aUu6SizG4y-maGR0cDSm8I5F?usp=drive_link

Conclusion

The Greenhouse Monitoring System was successfully implemented using an Arduino UNO. It demonstrated the use of sensors, PWM, LCD, and UART communication in a real-world automation scenario. The system supports both automatic and manual control, making it versatile for various use cases in agriculture and environmental monitoring.

Future Enhancements

- Add humidity-based fan control

- Include data logging to SD card

- Implement wireless control using Bluetooth (HC-05)

- Use a relay and heater module for real heating control

- Display temperature trend using graphical LCD or OLED

Submitted by

S. Mathan Raj
Pre-Final Year Engineering Student
Email: rajmathan7034@gmail.com
Project: Greenhouse Monitoring System using DHT11 and PWM Control